SIMATS ENGINEERING

Department of Computer Science and Engineering
ASSESSMENT TOOL 2 - CONCEPT MAPPING

| Course Code: | CSA1221 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO1 | Assessment: | ASSESSMENT TOOL 2 - CONCEPT MAPPING |
| Weightage: | 35% | Total Marks: | 25 |
| CO1 Statement: | Analyze the functional units of a computer system including CPU, memory, bus structures, and I/O components by examining instruction execution cycles, addressing modes, and performance metrics such as CPI, clock cycles, and benchmarking (BL4) |  |  |
| Student Name: | _Shree Aishwarya R_______ | Reg. No. / Date: | _______________________ |

ANNEXURE A — CONCEPT MAPPING SOLUTIONS

Question 1: System Architecture, Bus Interconnects & Signal Flows

Prompt: Construct a detailed concept map showing the relationship between CPU (ALU, Control Unit, Registers), memory (cache, RAM), and I/O devices. Extend the map to include single-bus, multi-bus, and hierarchical bus structures, and clearly illustrate how data and control signals flow among these components during execution. Indicate possible points of delay or bottlenecks and how different bus structures help in improving performance.

1. Architectural Concept Map & Relationship Topology

• Central Processing Unit (CPU): Houses the Control Unit (CU) which orchestrates control signals, the Arithmetic Logic Unit (ALU) for data processing, and Registers (PC, MAR, MDR, IR, ACC) for high-speed temporary storage.

• Memory Hierarchy: Comprises Level 1/2/3 Caches (SRAM) for immediate instruction/data buffering and Main Memory (RAM/DRAM) for primary program execution space.

• I/O Subsystem: Connects external peripherals via I/O Interfaces, Status/Control Registers, and Data Buffers using Memory-Mapped I/O or Isolated I/O schemes.

CONCEPT MAP TOPOLOGY — SYSTEM COMPONENT INTERCONNECTIONS +---------------------------------------------------------------------------------+ ¦ CPU CORE ¦ ¦ +--------------------+ +-----------------------+ +----------------------+ ¦ ¦ ¦ Control Unit (CU) ¦-->¦ Registers (PC, MAR..)¦-->¦ ALU (Data Execution)¦ ¦ ¦ +---------+----------+ +-----------+-----------+ +----------+-----------+ ¦ +------------¦--------------------------¦--------------------------¦--------------+ ¦ Control Signals ¦ Address/Data ¦ Data Result v v v ================================ SYSTEM BUS / BUS HIERARCHY ===================== ¦ Control Bus (READ/WRITE/ACK) ¦ Address Bus (A0-An) ¦ Data Bus (D0-Dn) ================================================================================= ¦ ¦ ¦ +-------v--------------------------v--------------------------v-------+ ¦ MEMORY SUB-SYSTEM ¦ ¦ +---------------------------+ +----------------------------+ ¦ ¦ ¦ L1/L2 Cache Memory (SRAM) ¦ <---> ¦ Main Memory (DRAM/RAM) ¦ ¦ ¦ +---------------------------+ +----------------------------+ ¦ +----------------------------------------------------------------------+ ¦ +---------v----------+ ¦ I/O SYSTEM ¦ ¦ (Controllers/Devs) ¦ +--------------------+

2. Bus Structures Comparison & Signal Dynamics

| Bus Architecture | Structure Overview | Data & Control Signal Flow | Performance Impact & Throughput |
| --- | --- | --- | --- |
| Single-Bus Architecture | All components (CPU, Memory, I/O) share a single common system bus (Address, Data, Control lines). | Time-multiplexed. Only one device pair can transfer data at any given clock cycle. | Severe bottleneck (Bus Contention). Low cost, low throughput. Performance drops sharply with multiple devices. |
| Multi-Bus Architecture | Separate buses dedicated to specific component pairs (e.g., Memory Bus, System Bus, I/O Bus). | Parallel transfer allowed. CPU can read from Cache/RAM while I/O operates on local I/O bus. | Reduced contention, higher overall systems bandwidth, increased hardware layout complexity. |
| Hierarchical Bus Architecture | Tiered organization: Local Bus (CPU-L1), System/Memory Bus (L2/L3-RAM), Expansion/PCIe Bus (I/O). Connected via Host & PCI Bridges. | High-speed data stays on top-tier buses. Transfers pass through bridges with signal translation & buffering. | Maximum performance. Prevents slow I/O devices from degrading high-speed CPU-Memory transfers. |

3. Bottlenecks, Delays & Optimization Mechanisms

• Von Neumann Bottleneck: Shared bus for instructions and data creates a throughput limit because the CPU cannot read an instruction and read/write data simultaneously.

• Propagation & Bus Arbitration Delay: Physical length of bus traces and control logic overhead to resolve master/slave access conflicts add latency.

• Memory Latency & Wait States: DRAM access times are significantly slower than CPU clock speed, forcing CPU insertion of wait states.

• Mitigation via DMA & Cache Hierarchies: Direct Memory Access (DMA) offloads bulk I/O transfer from CPU; Multi-level SRAM Caches keep frequent operations on high-speed local buses.

KEY ARCHITECTURAL INSIGHT Hierarchical bus structures overcome single-bus contention by segmenting high-speed CPU memory traffic from low-speed peripheral transfers using bridges, maximizing overall system bandwidth.

Question 2: Instruction Execution Cycle & Performance Metrics

Prompt: Develop a comprehensive concept map linking the instruction execution cycle (fetch, decode, execute) with CPU performance parameters such as CPI, clock cycles, and execution time. Show how each stage contributes to overall execution time, and include connections to factors like memory access, instruction complexity, and pipeline stalls. Also, represent how benchmarking is used to evaluate performance.

1. Instruction Cycle Dynamics & Stage Breakdown

| Cycle Stage | Hardware Operations | Delay Factors / Bottlenecks | Impact on Performance Metrics |
| --- | --- | --- | --- |
| Fetch (IF) | PC -> MAR -> Address Bus -> Memory Read -> MDR -> IR; PC incremented. | Cache misses, memory latency, bus contention. | Increases Memory Access Penalty, adding clock cycles per instruction (CPI). |
| Decode (ID) | Control Unit decodes opcode in IR, identifies operands, reads register file. | Complex Instruction Decoding (CISC variable length). | Increases pipeline depth and clock cycle time ($T_{clk}$). |
| Execute (EX) | ALU performs operation; or memory address calculated for load/store. | Complex arithmetic (multiplication, division), ALU latency. | Requires multiple execution clock cycles if non-pipelined. |
| Writeback (WB) | Result written back to Register File or Memory. | Memory write buffer full, bus availability. | Causes structural stalls or hazards in pipeline. |

2. CPU Performance Equation & Parameter Linkage

• Fundamental Performance Equation: CPU Execution Time = Instruction Count (IC) x Cycles Per Instruction (CPI) x Clock Cycle Time (Clock Cycle Time = 1 / Frequency).

• Instruction Complexity Impact: Complex instructions (CISC) reduce Instruction Count (IC) but increase Cycles Per Instruction (CPI). Simple instructions (RISC) increase IC but yield CPI close to 1.0.

• Pipeline Stalls (Hazards): Stalls delay instruction flow. Total CPI = CPI_ideal + Stall_structural + Stall_data + Stall_control.

CPU PERFORMANCE CONCEPT MAP METRIC FLOW +-----------------------+ +------------------------+ +------------------------+ ¦ Instruction Count ¦ ¦ Cycles Per Instruction ¦ ¦ Clock Cycle Time (T) ¦ ¦ (IC) ¦ ¦ (CPI) ¦ ¦ (1 / Frequency) ¦ +-----------+-----------+ +-----------+------------+ +-----------+------------+ ¦ ¦ ¦ Software / Compiler Pipeline Stalls & Hardware Design & ISA Architecture Memory Latency Impact Transistor Technology ¦ ¦ ¦ +----------------------+ ¦ +-----------------------+ v v v +---------------------------------------+ ¦ CPU EXECUTION TIME = IC x CPI x T ¦ +-------------------+-------------------+ ¦ v +---------------------------------------+ ¦ BENCHMARKING & EVALUATION (SPEC) ¦ ¦ Normalized Execution Ratios ¦ +---------------------------------------+

3. Benchmarking & Real-World Evaluation

• Role of Benchmarks: Synthetically tests performance across standard workloads (e.g., SPEC CPU2017, MIPS, MFLOPS) rather than relying on raw clock frequency alone.

• Geometric Mean Normalization: SPEC ratio = Execution Time (Reference Computer) / Execution Time (Test Computer). Overall performance is summarized via Geometric Mean across benchmark suites.

PERFORMANCE EQUATION INSIGHT Overall CPU execution performance is a balance of three distinct factors: Compiler optimization (IC), Microarchitecture design (CPI), and Semiconductor physical speed (Clock Period).

Question 3: Architectural Comparison — 8085 vs. 8086

Prompt: Create a concept map comparing 8085 and 8086 architectures, including their instruction sets, register organization, addressing modes, and memory segmentation (in 8086). Clearly show how these architectural features influence program execution, multitasking capability, and efficiency in real-time applications.

1. Comparative Matrix (8085 vs. 8086)

| Architectural Feature | Intel 8085 Microprocessor | Intel 8086 Microprocessor |
| --- | --- | --- |
| Data & Address Bus Width | 8-bit Data Bus, 16-bit Address Bus (64 KB address space). | 16-bit Data Bus, 20-bit Address Bus (1 MB address space). |
| Register Organization | 8-bit Accumulator (A), B, C, D, E, H, L registers, Stack Pointer (SP), Program Counter (PC). | 16-bit General Registers (AX, BX, CX, DX), Pointer/Index Registers (SP, BP, SI, DI), Segment Registers (CS, DS, SS, ES). |
| Memory Architecture | Flat, linear memory space up to 64 KB. | Segmented memory (1 MB divided into 64 KB segments via Segment:Offset addressing). |
| Pipelining & Execution | Single Execution Unit (EU). Fetch and Execute occur sequentially (no pipelining). | Dual units: Bus Interface Unit (BIU) with 6-byte instruction queue and Execution Unit (EU) operating in parallel. |
| Addressing Modes | 5 Addressing Modes (Direct, Immediate, Register, Register Indirect, Implicit). | 24+ Addressing Modes (including Based Indexed, Scaled, Relative). |
| Multitasking & Real-Time Support | Very limited. Flat memory lacks hardware memory protection; unsuitable for true multitasking. | Hardware segmentation allows dynamic code relocation, task isolation, and efficient multitasking execution. |

2. Memory Segmentation in 8086

• Physical Address Calculation: Physical Address = (Segment Register x 16) + Offset Address [or (Segment Register << 4) + Offset].

• Segment Registers: CS (Code Segment), DS (Data Segment), SS (Stack Segment), ES (Extra Segment) enable code separation and modular memory management.

• Multitasking & Execution Advantage: Programs can be relocated in memory simply by updating the segment registers without modifying instruction offset addresses.

ARCHITECTURAL EVOLUTION The introduction of the Bus Interface Unit (BIU) instruction queue and Memory Segmentation in the 8086 marked the transition from basic microcontroller-like execution to pipelined, multitasking-capable architecture.

Question 4: Addressing Modes & Instruction Type Mapping

Prompt: Draw a detailed concept map that connects various addressing modes (immediate, direct, register, indirect, indexed) with instruction types (data transfer, arithmetic, control instructions). Illustrate how each addressing mode affects memory access time, instruction size, and execution speed, and show real-world scenarios where specific addressing modes are preferred.

1. Addressing Modes Comprehensive Analysis Matrix

| Addressing Mode | Operand Location | Memory Accesses | Instruction Size | Execution Speed | Real-World Best Application |
| --- | --- | --- | --- | --- | --- |
| Immediate | Within Instruction | 0 Memory Reads | Medium (Includes Operand) | Fastest | Initializing constants/counters (e.g., MOV AX, 0005H). |
| Direct | Address in Instruction | 1 Memory Read | Large (Includes Address) | Moderate | Accessing global variables with fixed addresses. |
| Register | In CPU Register | 0 Memory Reads | Smallest (Register Code) | Extremely Fast | High-speed arithmetic (e.g., ADD AX, BX). |
| Register Indirect | Address in Register | 1 Memory Read | Small | Fast | Pointer dereferencing, dynamic memory traversal. |
| Indexed / Based | Base/Index Reg + Disp | 1 Memory Read | Medium to Large | Moderate | Array indexing, struct field access, table lookups. |

2. Instruction Type Mapping

• Data Transfer Instructions (MOV, PUSH, POP, IN, OUT): Utilize Immediate, Register, Direct, and Indirect modes to move data between CPU registers, memory locations, and I/O ports.

• Arithmetic & Logical Instructions (ADD, SUB, AND, XOR): Prefer Register and Immediate modes for maximum speed and minimal execution latency.

• Control Flow Instructions (JMP, CALL, RET, JZ): Utilize Relative and Indirect addressing modes to achieve branching and function call routines.

| ADDRESSING MODES TO INSTRUCTION TYPES CONCEPT MAP +-------------------------------------------------------------------------+ ¦ ADDRESSING MODES ¦ ¦ [Immediate] [Register] [Direct] [Reg-Indirect] [Indexed] ¦ +------+--------------+--------------+---------------+--------+-----------+ ¦ ¦ ¦ ¦ ¦ ¦ Constant ¦ Speed ¦ Absolute ¦ Ptr ¦ Array/Struct v v v v v +--------------+--------------+--------------+---------------+------------+ ¦ DATA TRANSFER¦ ARITHMETIC ¦ CONTROL ¦ MEMORY IMPACT ¦ CODE SIZE ¦ ¦ (MOV, PUSH) ¦ (ADD, SUB) ¦ (JMP, CALL) ¦ Access Time ¦ Length ¦ +--------------+--------------+--------------+---------------+------------+ |  |
| --- | --- |
| DESIGN TRADE-OFF INSIGHT Selecting the appropriate addressing mode trades off code density (bytes) against execution speed (clock cycles) and flexibility (dynamic indexing). |

Question 5: Number Systems, Instruction Encoding & Debugging

Prompt: Prepare an elaborate concept map integrating number systems (binary and hexadecimal) with instruction representation, memory storage, and debugging processes. Show how data is converted between formats, how instructions are stored and interpreted by the CPU, and why hexadecimal representation is commonly used in low-level programming and system design.

1. Data Representation & Conversion Pipeline

• Binary (Base-2): The fundamental hardware machine language level (0s and 1s representing voltage levels LOW and HIGH).

• Hexadecimal (Base-16): Human-readable compact representation of binary where 1 Hex digit corresponds exactly to 4 Binary bits (1 Nibble).

• Conversion Mapping: Example: Binary 1011 0000 0100 0101_2 maps directly to Hex B045_16 without complex division/multiplication.

2. Instruction Encoding & Memory Storage Mechanisms

• Instruction Encoding: Assembly instructions translate directly into Machine Code Opcode + Operands (e.g., 'MOV AL, 05H' -> B0 05 in Hex).

• Byte Ordering (Endianness): Little-Endian stores the Least Significant Byte (LSB) at lower memory address; Big-Endian stores Most Significant Byte (MSB) first.

| Assembly Instruction | Hex Machine Code | Binary Representation | Memory Layout (Little-Endian) |
| --- | --- | --- | --- |
| MOV AL, 45H | B0 45 | 10110000 01000101 | Addr [0000]: B0, Addr [0001]: 45 |
| MOV AX, 1234H | B8 34 12 | 10111000 00110100 00010010 | Addr [0000]: B8, Addr [0001]: 34 (LSB), Addr [0002]: 12 (MSB) |
| ADD AX, BX | 01 B8 | 00000001 10111000 | Addr [0000]: 01, Addr [0001]: B8 |

3. Hexadecimal in Debugging & System Design

• Readability & Compactness: Reduces 32-bit binary strings (32 digits) to 8 hex digits, preventing human operator transcription error during memory dumps.

• Memory Dumps & Pointer Alignment: Hex aligns perfectly with byte boundaries (2 hex chars = 1 byte), making memory offset calculation straightforward.

• Role in Debugging (GDB / SoftICE): Debuggers present memory dumps, disassembly registers, and stack pointers in Hex for quick inspection of corrupted addresses and stack overflows.

| NUMBER SYSTEMS, ENCODING & DEBUGGING CONCEPT MAP +--------------------------+ +--------------------------+ ¦ BINARY DATA / SIGNALS ¦ <-------> ¦ HEXADECIMAL FORMAT ¦ ¦ (Hardware Logic Levels) ¦ 1-to-4 Bit¦ (Compact Human Notation) ¦ +------------+-------------+ Mapping +------------+-------------+ ¦ ¦ v v +--------------------------+ +--------------------------+ ¦ INSTRUCTION ENCODING ¦ ¦ MEMORY DUMP / SP ¦ ¦ (Opcode + Operands) ¦ ¦ (Little/Big Endian) ¦ +------------+-------------+ +------------+-------------+ ¦ ¦ +-------------------+------------------+ v +----------------------------------+ ¦ SYSTEM DEBUGGING & DIAGNOSTICS ¦ ¦ (Disassembly, Stack Inspection)¦ +----------------------------------+ |  |
| --- | --- |
| SYSTEM DESIGN CONCLUSION Hexadecimal representation bridges raw binary hardware implementation with human cognitive comprehension, providing direct bitwise alignment essential for low-level debugging and architecture design. |

Code of Conduct:

I ________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: ________________________