SIMATS ENGINEERING

ASSESSMENT TOOL 1 | COMPREHENSIVE SOLUTIONS

| COURSE NAME: Computer Architecture | COURSE CODE: CSA1221 |
| --- | --- |
| COURSE OUTCOME: CO1 (Analyze functional units, bus structures, execution cycles, addressing modes & performance metrics) | ASSESSMENT WEIGHTAGE: 40% |
| TOTAL MARKS: 25 Marks | COGNITIVE LEVEL: Bloom's Level 4 (Analyze) |

QUESTION 1: Smart City Surveillance System Bottleneck & Architecture Analysis

A smart city surveillance system processes continuous video feeds from multiple cameras, where large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours, the system experiences lag and delayed response in threat detection. Analyze how the interaction between CPU, memory, and I/O contributes to this issue, and explain how different bus structures (single-bus vs multi-bus) and improvements in instruction execution cycle can enhance system performance.

Detailed Solution & Analysis:

A smart city surveillance system continuously receives video streams from multiple cameras. The CPU, memory, and I/O devices work together to process this large amount of data. During peak hours, delays occur because of heavy data transfer and processing demands.

1. Interaction between CPU, Memory, and I/O

- I/O Devices (Cameras): Continuously capture and send high-resolution video data to the system.

- Memory (RAM): Temporarily stores incoming video frames before processing.

- CPU: Fetches video data from memory, executes image processing and threat detection algorithms, and sends results to storage or monitoring systems.

During peak traffic:

- Many cameras send data simultaneously.

- The CPU spends extra time waiting for data from memory or I/O devices.

- Memory becomes overloaded with continuous video frames.

- The bus connecting CPU, memory, and I/O becomes congested, causing delays in data transfer.

- Threat detection responses become slower.

2. Effect of Bus Structures

(a) Single-Bus Structure

- A single communication bus is shared by the CPU, memory, and all I/O devices.

- Only one data transfer can occur at a time.

- Simultaneous requests from multiple cameras create bus contention.

- The CPU waits for access to the bus, reducing overall system performance.

Advantages:

- Simple design.

- Low cost.

Disadvantages:

- Bus becomes a bottleneck.

- Higher waiting time during heavy workloads.

- Poor performance for real-time surveillance.

(b) Multi-Bus Structure

- Uses separate buses for CPU-memory communication and I/O communication.

- Multiple data transfers can occur simultaneously.

- Reduces bus congestion and waiting time.

- Improves throughput and allows faster processing of video streams.

Advantages:

- Faster communication.

- Higher bandwidth.

- Better support for multiple cameras.

- Improved real-time threat detection.

Disadvantages:

- More expensive.

- More complex hardware design.

3. Improvements in the Instruction Execution Cycle

The CPU instruction cycle consists of Fetch → Decode → Execute → Memory Access → Write Back.

Performance can be improved by:

- Instruction Pipelining: Executes multiple instructions simultaneously in different stages, increasing throughput.

- Cache Memory: Stores frequently used data and instructions, reducing memory access time.

- DMA (Direct Memory Access): Allows I/O devices to transfer data directly to memory without continuous CPU involvement, reducing CPU workload.

- Parallel Processing/Multi-Core CPUs: Different cores process multiple video streams simultaneously.

- Branch Prediction and Out-of-Order Execution: Reduce idle CPU cycles and improve instruction execution efficiency.

4. Overall Performance Improvement

By replacing a single-bus architecture with a multi-bus architecture and improving the instruction execution cycle through pipelining, caching, DMA, and parallel processing:

- Data transfer between CPU, memory, and I/O becomes faster.

- Bus bottlenecks are minimized.

- CPU idle time is reduced.

- More video streams can be processed simultaneously.

- Threat detection becomes faster and more accurate.

- Overall system responsiveness and reliability improve during peak hours.

Conclusion

The lag in the surveillance system is mainly caused by heavy data traffic between I/O devices, memory, and the CPU, leading to bus congestion and CPU waiting time. A multi-bus architecture enables simultaneous communication and reduces bottlenecks, while enhancements such as pipelining, cache memory, DMA, and multi-core processing improve instruction execution efficiency. Together, these improvements significantly increase system performance and ensure timely threat detection in smart city surveillance applications.

QUESTION 2: Stock Trading Application CPI Reduction & CPU Performance Enhancement

A real-time stock trading application running on a processor executes millions of instructions per second, but users report delays in transaction processing during high load. The system has a high CPI due to frequent memory access and branch instructions. Examine how the fetch-decode-execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and improve overall CPU performance using suitable architectural enhancements.

Answer: Fetch-Decode-Execute Cycle and CPU Performance in a Real-Time Stock Trading Application

A real-time stock trading application executes millions of instructions every second. During high load, users experience delays because the processor has a high CPI (Cycles Per Instruction) caused by frequent memory access and branch instructions.

1. Impact of the Fetch-Decode-Execute Cycle on Execution Time

The CPU executes each instruction through the following stages:

- Fetch: The CPU retrieves the instruction from memory.

- Decode: The control unit interprets the instruction.

- Execute: The ALU or execution unit performs the required operation.

- Memory Access: Data is read from or written to memory if needed.

- Write Back: The result is stored in registers or memory.

During heavy trading activity:

- Frequent memory access increases the time spent in the fetch and memory access stages.

- Branch instructions cause pipeline interruptions because the CPU may fetch the wrong instructions.

- The CPU spends more clock cycles completing each instruction, resulting in a high CPI.

- Increased execution time delays transaction processing and reduces system performance.

2. Methods to Reduce CPI

a) Cache Memory

- Stores frequently used instructions and data close to the CPU.

- Reduces memory access time.

- Lowers the number of clock cycles required per instruction.

b) Instruction Pipelining

- Executes multiple instructions simultaneously in different stages.

- Improves instruction throughput.

- Reduces average CPI.

c) Branch Prediction

- Predicts the outcome of branch instructions before execution.

- Minimizes pipeline stalls caused by incorrect branching.

- Improves execution efficiency.

d) Out-of-Order Execution

- Executes independent instructions while waiting for slower instructions to complete.

- Reduces CPU idle time.

- Improves processor utilization.

e) Multi-Level Cache (L1, L2, L3)

- Provides faster access to frequently used data.

- Reduces delays caused by main memory access.

- Improves overall execution speed.

3. Architectural Enhancements

- Multi-Core Processors: Execute multiple trading requests simultaneously.

- Superscalar Architecture: Executes more than one instruction per clock cycle.

- DMA (Direct Memory Access): Transfers data between memory and I/O devices without constant CPU involvement.

- High-Speed Memory (DDR5 or Similar): Reduces memory latency and improves data transfer speed.

- Wider Data Bus and Multi-Bus Architecture: Allows faster communication between CPU, memory, and I/O devices, reducing bottlenecks.

4. Overall Performance Improvement

By reducing CPI through cache memory, pipelining, branch prediction, and out-of-order execution, along with architectural enhancements such as multi-core processors and multi-bus systems:

- Memory access delays are minimized.

- Pipeline stalls are reduced.

- More instructions are completed per second.

- Transaction processing becomes faster.

- CPU throughput and system responsiveness improve during high trading loads.

Conclusion

The fetch-decode-execute cycle directly affects execution time in a real-time stock trading application. Frequent memory access and branch instructions increase CPI, causing slower transaction processing. Techniques such as cache memory, instruction pipelining, branch prediction, out-of-order execution, multi-core processors, and multi-bus architecture reduce CPI and improve CPU performance, ensuring faster and more efficient transaction processing during peak workloads.

QUESTION 3: Embedded 8085 Industrial Temperature Control Addressing Modes Analysis

An embedded system based on an 8085 microprocessor is used in an industrial temperature control unit, where sensors provide input and actuators respond accordingly. However, incorrect temperature readings are occasionally processed due to improper use of addressing modes in the program. Discuss how different addressing modes influence data access, identify possible causes of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure accurate and reliable operation.

Answer: Addressing Modes and Instruction Set of 8085 in an Industrial Temperature Control Unit

An industrial temperature control unit based on the 8085 microprocessor receives temperature data from sensors and controls actuators accordingly. Incorrect temperature readings may occur if the program uses inappropriate addressing modes, resulting in wrong data being processed.

1. Influence of Addressing Modes on Data Access

The 8085 microprocessor supports different addressing modes that determine how data is accessed.

a) Immediate Addressing Mode

- Data is provided directly in the instruction.

- Example: MVI A, 25H

- Suitable for loading fixed values or constants.

b) Register Addressing Mode

- Data is stored in CPU registers.

- Example: MOV A, B

- Provides fast data access because registers are inside the CPU.

c) Direct Addressing Mode

- The memory address of the data is specified in the instruction.

- Example: LDA 2500H

- Used when the sensor data is stored at a fixed memory location.

d) Register Indirect Addressing Mode

- The memory address is stored in a register pair (HL, BC, or DE).

- Example: MOV A, M

- Useful for accessing data stored in different memory locations.

e) Implied Addressing Mode

- The operand is implied by the instruction.

- Example: CMA, RAL

- No explicit data address is required.

2. Possible Causes of Incorrect Temperature Readings

- Using the wrong memory address while reading sensor data.

- Incorrect use of register indirect addressing, causing access to the wrong memory location.

- Register values being overwritten before use.

- Failure to initialize register pairs correctly.

- Programming errors in data transfer instructions.

- Noise or invalid sensor data not being checked before processing.

3. Effective Use of the 8085 Instruction Set

The following instructions help ensure accurate and reliable operation:

- Data Transfer Instructions

  - MOV, MVI, LDA, STA, LHLD, SHLD

  - Transfer sensor readings correctly between memory and registers.

- Arithmetic Instructions

  - ADD, ADI, SUB, INR, DCR

  - Perform temperature calculations and calibration.

- Logical Instructions

  - ANA, ORA, XRA, CMP

  - Compare temperature values with preset limits.

- Branch Instructions

  - JZ, JNZ, JC, JNC, JMP

  - Execute control actions based on temperature conditions.

- Input/Output Instructions

  - IN, OUT

  - Read sensor data and send control signals to actuators.

4. Methods to Ensure Accurate Operation

- Use the correct addressing mode for each memory operation.

- Initialize registers and memory addresses before accessing data.

- Validate sensor readings before processing.

- Store important values safely to prevent accidental overwriting.

- Use conditional branch instructions to detect abnormal readings.

- Test and debug the program thoroughly before deployment.

Conclusion

Addressing modes determine how the 8085 microprocessor accesses data from memory and registers. Incorrect use of these modes can lead to wrong temperature readings and unreliable control. By selecting the appropriate addressing mode and effectively using the 8085 instruction set for data transfer, arithmetic, logical operations, branching, and I/O, the industrial temperature control unit can achieve accurate, reliable, and efficient operation.

QUESTION 4: 8086 Microprocessor Segmented Memory Management & Error Mitigation

A computing system uses an 8086 microprocessor with segmented memory for executing multiple programs simultaneously, but it encounters issues like incorrect data access and stack overflow errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how improper segment handling can lead to such problems, and explain how instruction execution and addressing modes must be managed to ensure correct program execution.

Answer: Segmentation, Instruction Execution, and Addressing Modes in 8086

An 8086 microprocessor uses segmented memory to organize memory efficiently and support multiple programs. If segments are not handled correctly, problems such as incorrect data access and stack overflow can occur.

1. Segmentation in 8086

The 8086 divides memory into different segments, each with a maximum size of 64 KB.

a) Code Segment (CS)

- Stores program instructions.

- The Code Segment (CS) register holds the base address.

- The Instruction Pointer (IP) points to the next instruction.

b) Data Segment (DS)

- Stores variables, constants, and program data.

- The Data Segment (DS) register points to the data area.

- Most data operations use DS by default.

c) Stack Segment (SS)

- Stores temporary data, function calls, and return addresses.

- The Stack Segment (SS) register and Stack Pointer (SP) manage the stack.

- The stack follows the Last In, First Out (LIFO) principle.

2. Problems Due to Improper Segment Handling

- Incorrect initialization of segment registers (CS, DS, SS) may cause the CPU to access the wrong memory locations.

- Using an incorrect data segment results in invalid data being read or written.

- Stack overflow occurs when too many values are pushed onto the stack without corresponding POP operations.

- Incorrect stack pointer values can overwrite important memory or program data.

- Jumping to the wrong code segment may cause execution of invalid instructions.

- Failure to update segment registers after program switching can lead to memory access errors.

3. Instruction Execution in 8086

The instruction execution cycle consists of:

- Fetch: The CPU fetches the instruction from the Code Segment using CS:IP.

- Decode: The instruction is decoded by the control unit.

- Execute: The required operation is performed.

- Memory Access: Data is read from or written to memory if required.

- Write Back: The result is stored in registers or memory.

Correct segment register values ensure that instructions and data are fetched from the proper memory locations.

4. Addressing Modes in 8086

The 8086 supports several addressing modes:

- Immediate Addressing: Data is included in the instruction.
Example: MOV AX, 1234H

- Register Addressing: Operands are stored in registers.
Example: MOV AX, BX

- Direct Addressing: Memory address is specified directly.
Example: MOV AX, [2500H]

- Register Indirect Addressing: Memory address is stored in a register.
Example: MOV AX, [BX]

- Based and Indexed Addressing: Uses base and index registers for accessing arrays and data structures.
Example: MOV AX, [BX+SI]

Using the correct addressing mode ensures accurate memory access and prevents data errors.

5. Ensuring Correct Program Execution

- Initialize CS, DS, SS, and ES correctly before execution.

- Keep the Stack Pointer (SP) within the stack limits.

- Match every PUSH instruction with a corresponding POP instruction.

- Use appropriate addressing modes for accessing memory.

- Save and restore segment registers during program switching.

- Validate memory addresses before accessing data.

- Test and debug segment-related operations thoroughly.

Conclusion

The 8086 segmented memory architecture separates program instructions, data, and stack into Code, Data, and Stack segments. Improper handling of segment registers can cause incorrect memory access, program errors, and stack overflow. By correctly managing segment registers, instruction execution, stack operations, and addressing modes, the system can execute multiple programs accurately, efficiently, and reliably.

QUESTION 5: Hexadecimal/Binary Conversion, Instruction Execution Impact & Benchmarking

A data communication system represents packet information in hexadecimal format for ease of debugging and transmission, but a software engineer mistakenly interprets values, leading to incorrect data processing. Analyze the importance of number system conversions between hexadecimal and binary in such systems, explain how errors in conversion can affect instruction execution, and discuss how efficient CPU design and benchmarking can help detect and minimize such issues in real-time systems.

Answer: Number System Conversion, Instruction Execution, CPU Design, and Benchmarking

A data communication system represents packet information in hexadecimal format because it is compact and easy to read. Since computers process data in binary, accurate conversion between hexadecimal and binary is essential. Incorrect conversion can lead to wrong data interpretation and processing errors.

1. Importance of Hexadecimal to Binary Conversion

- Computers execute instructions only in binary (0s and 1s).

- Hexadecimal provides a shorter and more readable representation of binary data.

- Each hexadecimal digit represents 4 binary bits.

- During debugging, engineers use hexadecimal values to examine memory contents, packet data, and machine instructions.

- Correct conversion ensures that packet information is interpreted accurately during transmission and processing.

Example:

- Hexadecimal: 3A

- Binary: 00111010

If the conversion is incorrect, the CPU may interpret the wrong data or instruction.

2. Effects of Conversion Errors on Instruction Execution

The CPU executes instructions through the following stages:

- Fetch: Retrieves the instruction from memory.

- Decode: Interprets the binary instruction.

- Execute: Performs the required operation.

- Memory Access: Reads or writes data if necessary.

- Write Back: Stores the result.

If hexadecimal values are converted incorrectly:

- The CPU may fetch incorrect instructions.

- Wrong operands may be decoded.

- Incorrect arithmetic or logical operations may be performed.

- Memory addresses may become invalid.

- Packet data may be corrupted during processing.

- The system may produce incorrect outputs or fail during execution.

3. Efficient CPU Design to Minimize Errors

Modern CPU architectures improve reliability through:

- Cache Memory: Provides faster access to frequently used instructions and data.

- Instruction Pipelining: Executes multiple instructions simultaneously, increasing throughput.

- Error Detection Mechanisms: Detect invalid instructions or corrupted data.

- Efficient Control Unit: Ensures proper instruction decoding and execution.

- High-Speed Buses: Improve communication between CPU, memory, and I/O devices.

- Memory Protection: Prevents unauthorized or incorrect memory access.

These features reduce processing errors and improve overall system performance.

4. Role of Benchmarking

Benchmarking evaluates CPU performance under different workloads.

It helps to:

- Measure instruction execution speed.

- Detect delays caused by incorrect data processing.

- Compare processor performance using standard benchmark programs.

- Identify memory and CPU bottlenecks.

- Evaluate improvements in hardware and software.

- Ensure reliable performance in real-time communication systems.

Common performance metrics include:

- Execution Time

- Clock Speed

- CPI (Cycles Per Instruction)

- MIPS (Million Instructions Per Second)

- Throughput

5. Ensuring Reliable Real-Time Operation

- Verify hexadecimal-to-binary conversions before processing.

- Use debugging tools to validate packet contents.

- Implement error detection and correction techniques.

- Use efficient CPU architectures with cache and pipelining.

- Perform regular benchmarking to identify performance issues.

- Test communication software thoroughly under heavy workloads.

Conclusion

Hexadecimal-to-binary conversion is essential because CPUs execute instructions only in binary form. Incorrect conversions can lead to invalid instruction execution, incorrect memory access, and data corruption. Efficient CPU design, including cache memory, pipelining, and error detection, together with benchmarking techniques, helps detect performance problems, reduce processing errors, and ensure accurate and reliable operation in real-time data communication systems.